# Popular 3D Printing Software:

## Design

- TinkerCad, Blender, Fusion

## Slicers

- Cura, Pursa

### Steps to Printing a 3D Model

- Design or download your model (STL file).

- Open your slicer and “slice the print.” This will give you a gcode file.

- Add the gcode file to your memory card.

- Put the memory card in the printer, and heat up.

- For PLA Filament, I have the nozzle at 200℃ and the hotbed at 60℃.

- After the printer is heated up, start your print!

- Note: Purge lines and skirt will print first (Depending on settings)

### Tips When 3D Printing

- Increase the temperature for smaller prints to help them stick to the hotbed.

  - Adding a brim, raft, or skirt to the print can also help a smaller print stick to the bed.

- Any oil, dust, etc. on the hotbed can cause prints to come unstuck…

  - Clean bed with dawn and hot, hot water once a week (helps prints stick)

  - Wipe down the hotbed with rubbing alcohol and a cotton ball after each print (helps print stick)

- Save your failed prints and other filament scraps to melt down in the oven into silicone molds to make other manipulatives

- Use acrylic paint markers and Mod Podge sealer to add color and protection to your prints

### Places to get 3D Models

- Paths to Literacy General Link

- Round Table 3D Printing Link

- TacTiles 3D Printing for VI:

  - O&M Crossings

  - School Yard

  - Shape Tower Game

- Thingiverse 3D Designs for Physical Objects

- Braille and Large Print Services 3D

- Braille and Print Blocks

- 3D Printing Braille

- Perkins School for the Blind

  - 3 D printing

  - Printing models

- 3D Models Donated

- 3D Models Databases (collections of all types of 3D models)

  - You Magazine

  - Thingiverse

  - Yeggi

  - Printables

  - Cults3d

  - My mini factory

  - Pinshape

- Assistive Writing/Drawing Guide

  - model 753450

- Models of the Eyes

  - model 381790

  - Thing:2833672

  - Thing:3117478

### TinkerCad Helpful Hints